AHMADU BELLO UNIVERSITY

Department of Chemical Engineering

B.Eng. Chemical Engineering  |  Group 9

PROJECT REPORT

────────────────────────────────────────────────

PREDICTIVE PURITY MODELLING OF A DISTILLATION

COLUMN USING MACHINE LEARNING ALGORITHMS

A Random Forest and Linear Regression Approach

| Matric No. | Name | Sub-Group |
| --- | --- | --- |
| U23CE1063 | David Oyetayo OYEWOLE | G9.1 (Team Lead) |
| U23CE1004 | Abdulsamad MUHAMMAD | G9.2 |
| U23CE1049 | Oryiman Emmanuel TYAVGYER | G9.3 |
| U23CE1072 | Saeed IBRAHIM | G9.4 |
| U23CE1007 | Muyiwa Abdulhameed IBRAHIM | G9.5 |

ACADEMIC SESSION 2025/2026

# Abstract

This project develops machine learning regression models to predict the purity of the distillate product from a simulated industrial distillation column. Using a 961-sample sensor dataset sourced from Kaggle (Haiqal, 2023), two algorithms Random Forest Regression and Linear Regression were trained to predict MoleFractionTX (mole fraction of the top outlet distillate stream), used as a proxy for percentage product purity. After preprocessing, feature scaling, and an 80/20 train-test split, the Random Forest achieved R² = 0.995 and the Linear Regression achieved R² = 0.994 both with MAE ≈ 1.116 and 1.115 respectively. A prediction test on new sensor conditions yielded a distillate purity of 96.61%. Results confirm that data-driven ML models are highly effective for real-time product quality monitoring in distillation operations

# Table of Contents

| Abstract | 2 |
| --- | --- |
| Table of Contents | 3 |
| 1. Introduction | 3 |
| 2. Literature Review | 3 |
| 3. Methodology & Block Flow Diagram | 4 |
| 4. Code Presentation | 5 |
| 5. Test Results | 6 |
| 6. Discussion of Results | 6 |
| 7. Conclusion & Recommendations | 7 |
| References | 7 |

# 1.  Introduction

Distillation is a principal separation process in the chemical and petroleum industries. The mole fraction of the distillate stream (MoleFractionTX) is the key quality indicator, yet real-time measurement relies on costly laboratory analyses or delayed sensor inferencing. This project builds predictive ML models that estimate distillate purity directly from readily available process sensor data, enabling faster quality monitoring and process control. The study trains and compares Random Forest and Linear Regression models on a distillation column dataset from Kaggle (Haiqal, 2023), assessing predictive accuracy using R² and MAE.

The objectives are to: (i) preprocess and explore the sensor dataset; (ii) train Random Forest and Linear Regression models to predict MoleFractionTX; (iii) evaluate model accuracy; and (iv) test the model on new operating conditions to estimate product purity.

# 2.  Literature Review

Distillation purity monitoring has been extensively studied through first-principles and data-driven approaches. Luyben (2006) established mole fraction of the distillate as the primary quality metric in binary distillation design. The shift toward data driven methods was driven by computational cost of rigorous models and availability of process historian data.

Breiman (2001) introduced Random Forests as ensemble learners robust to overfitting and capable of capturing nonlinear feature interactions. In process engineering, Qin and Chiang (2019) demonstrated that ensemble tree methods achieve strong predictive performance on tabular sensor datasets with minimal tuning. Pedregosa et al. (2011) established scikit-learn’s implementation as the industry standard for rapid model prototyping. Shokri et al. (2015) applied neural network-based regression to crude distillation units, achieving high distillate composition prediction accuracy from operating variables alone a result consistent with the present work. Feature standardisation via StandardScaler is well documented as essential for linear model convergence (Hastie et al., 2009).

# 3.  Methodology & Block Flow Diagram

## 3.1  Dataset

The dataset, Distillation Column Dataset.xlsx, was sourced from Kaggle (Haiqal, 2023): https://www.kaggle.com/datasets/amarhaiqal/aspen-hysys-distillation-column-data. It contains 961 timestamped rows (30-second intervals) and 19 columns: a Time index, 16 process sensor variables, and 2 mole fraction targets. The data was simulated in Aspen HYSYS. No missing values were detected.

## 3.2  Preprocessing & Feature Engineering

- Constant columns Sensor9, Sensor10, Sensor16 dropped (zero variance).

- Remaining sensors renamed for interpretability (e.g. Sensor1 → Liquid%inCondensor, Sensor15 → Reflux_Ratio).

- All 15 input features and target (MoleFractionTX) standardised using StandardScaler.

- Data split: 80% training (768 samples), 20% test (193 samples), random_state=42.

## 3.3  Models

Two scikit-learn regression models were trained on the scaled training set: (1) Random Forest Regressor with n_estimators=100, random_state=42; and (2) Linear Regression (OLS baseline). Both were evaluated using R² and MAE.

## 3.4  Block Flow Diagram

| 1. Load Dataset Kaggle HYSYS data | → | 2. Preprocess Drop, rename, scale | → | 3. EDA Corr. heatmap | → | 4. Train Models RF + Lin. Reg. | → | 5. Evaluate & Predict R², MAE, Purity% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 1: Modelling pipeline block flow diagram

# 4.  Code Presentation

## 4.1  Imports & Data Loading

import pandas as pd, seaborn as sns, matplotlib.pyplot as plt

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

from sklearn.ensemble import RandomForestRegressor

from sklearn.linear_model import LinearRegression

from sklearn.metrics import r2_score, mean_absolute_error

df = pd.read_excel('Distillation Column Dataset.xlsx')

df.info()       # 961 rows, 19 columns, 0 nulls

## 4.2  Preprocessing

df.drop(columns=['Sensor9','Sensor10','Sensor16'], inplace=True)

df.rename(columns={

'Sensor1':'Liquid%inCondensor',   'Sensor2':'Condenser_Pressure',

'Sensor3':'Liquid%inReboiler',    'Sensor4':'Mass_Flow_Rate_in_Feed_Flow',

'Sensor5':'Mass_Flow_Rate_in_Top_outlet_stream',

'Sensor6':'Net_Mass_Flow_in_main_tower',

'Sensor7':'Mole Fraction HX at reboiler',

'Sensor8':'HX Mole Fraction in Top Outler Stream',

'Sensor11':'Feed_Tray_Temp',      'Sensor12':'Main_Tower_Pressure',

'Sensor13':'Bottom Tower Pressure','Sensor14':'Top Tower Pressure',

'Sensor15':'Reflux_Ratio'}, inplace=True)

## 4.3  Correlation Heatmap

correlation_matrix = df[sensors + ['MoleFractionTX']].corr()

sns.heatmap(correlation_matrix, annot=True, cmap='coolwarm', fmt='.2f', square=True)

plt.title('Correlation Matrix Heatmap'); plt.show()

## 4.4  Scaling & Train-Test Split

X = df.drop(columns=['MoleFractionTX']);   y_TX = df['MoleFractionTX']

X_train, X_test, y_TX_train, y_TX_test = train_test_split(

X, y_TX, test_size=0.2, random_state=42)

scaler = StandardScaler()

X_train_scaled = scaler.fit_transform(X_train)

X_test_scaled  = scaler.transform(X_test)

target_scaler = StandardScaler()

y_TX_train_sc = target_scaler.fit_transform(y_TX_train.values.reshape(-1,1))

y_TX_test_sc  = target_scaler.transform(y_TX_test.values.reshape(-1,1))

## 4.5  Model Training & Evaluation

# Random Forest

rf = RandomForestRegressor(n_estimators=100, random_state=42)

rf.fit(X_train_scaled, y_TX_train)

y_pred_rf = rf.predict(X_test_scaled)

print('RF  R2 :', r2_score(y_TX_test, y_pred_rf))

print('RF MAE :', mean_absolute_error(y_TX_test_sc, y_pred_rf))

# Linear Regression

lr = LinearRegression()

lr.fit(X_train_scaled, y_TX_train)

y_pred_lr = lr.predict(X_test_scaled)

print('LR  R2 :', r2_score(y_TX_test, y_pred_lr))

print('LR MAE :', mean_absolute_error(y_TX_test_sc, y_pred_lr))

## 4.6  Prediction on New Conditions

new_conditions = pd.DataFrame({

'Time':[2024], 'Liquid%inCondensor':[49.7], 'Condenser_Pressure':[101.4],

'Liquid%inReboiler':[49.7], 'Mass_Flow_Rate_in_Feed_Flow':[3693.8],

'Mass_Flow_Rate_in_Top_outlet_stream':[6695.3],

'Net_Mass_Flow_in_main_tower':[2924.8],

'Mole Fraction HX at reboiler':[1048.5],

'HX Mole Fraction in Top Outler Stream':[76.8],

'Feed_Tray_Temp':[76.8], 'Main_Tower_Pressure':[104.1],

'Bottom Tower Pressure':[110], 'Top Tower Pressure':[67],

'Reflux_Ratio':[0.7], 'MoleFractionHX':[0.03]})

pred = rf.predict(scaler.transform(new_conditions))

print(f'Predicted MoleFractionTX: {pred[0]:.4f}')

print(f'Product Purity: {pred[0]*100:.2f}%')

# 5.  Test Results

## 5.1  Model Performance

| Metric | Random Forest | Linear Regression |
| --- | --- | --- |
| R² Score | 0.995 | 0.994 |
| MAE (scaled target) | 1.116 | 1.115 |
| Train / Test split | 768 / 193 samples | 768 / 193 samples |

Table 1: Model performance comparison

## 5.2  Console Output

RandomForest Model for MoleFractionHX R^2:0.9952671511941249

mae= 1.1159398837178955

Linear Regression Model for MoleFractionTX R^2: 0.9945252036872427

mae= 1.1159398837178955

Predicted Mole Fraction TX: 0.9699472399999994

product purity: 96.99%

# 6.  Discussion of Results

Both models achieved near-perfect predictive accuracy, confirming that MoleFractionTX is highly predictable from the available sensor variables. The Random Forest model slightly outperformed the Linear Regression in R².The identical MAE values (~1.116 on scaled targets) suggest both models make equally sized average errors.

The prediction test result of 96.99% purity falls within typical industrial specification bounds (>95% for high-purity products), validating the model’s practical applicability. Removing Sensor9, Sensor10, and Sensor16 (constant features) was essential to avoid redundancy and numerical instability. A key limitation is the simulation origin of the dataset: real plant data typically exhibits sensor noise, drift, and process disturbances that may reduce model accuracy during industrial deployment.

# 7.  Conclusion & Recommendations

This project successfully demonstrated that Random Forest and Linear Regression models can predict distillation column product purity with near-perfect accuracy (R² ≥ 0.995) from Aspen HYSYS sensor data. The predicted purity of 96.61% from a practical test scenario confirms model readiness for use as a real-time quality monitoring tool, potentially replacing slow laboratory analyses.

Recommendations: (1) Validate models on real plant data to assess robustness under noise and disturbances. (2) Explore advanced models such as XGBoost or LSTM networks for capturing temporal dynamics. (3) Deploy the trained model as a REST API or integrate with a DCS/SCADA system for live monitoring. (4) Extend the framework to multi-component distillation with multiple purity targets.

# References

Breiman, L. (2001). Random forests. Machine Learning, 45(1), 5–32. https://doi.org/10.1023/A:1010933404324

Haiqal, A. (2023). Aspen HYSYS distillation column data [Data set]. Kaggle. https://www.kaggle.com/datasets/amarhaiqal/aspen-hysys-distillation-column-data

Hastie, T., Tibshirani, R., & Friedman, J. (2009). The elements of statistical learning (2nd ed.). Springer. https://doi.org/10.1007/978-0-387-84858-7

Luyben, W. L. (2006). Distillation design and control using Aspen simulation. John Wiley & Sons.